Based on an equivalent model and synthetic storm: of 250 m average depth

Modified:

- Location of the new development. Added in new land cover characteristics

- Changed roughness values of proposed development area.

Table of results with a 250 mm rain event

| Scenario | Peak flow | Peak timing | Vol of flow | % Increase in Flow Vol over Base case |
| --- | --- | --- | --- | --- |
| Present Day LU | 3,856.5 | 840 mins | 32,624,637 |  |
| Low-intensity Developed | 3,839.4 | 840 mins | 32,672,969 | 0.148% |
| High intensity Developed | 3,754.1 | 840 mins | 32,767,408 | 0.438% |

Table of results with a 500 mm rain event

| Scenario | Peak flow | Peak timing | Vol of flow | % Increase in Flow Vol over Base case |
| --- | --- | --- | --- | --- |
| Present Day LU | 11,670 | 780 mins | 95,939,134 |  |
| Low-intensity Developed | 11,644 | 780 mins | 96,357,085 | 0.436% |
| High intensity Developed | 11,479 | 780 mins | 95,967,928 | 0.030% |

Hydrograph of current day land use

Hydrograph of  present day non-developed landscape mostly fallow grassland =  (Mannings N  of  0.34)

250 mm event

Hydrograph of  developed area with Low_impact development  (Mannings N  of  0.18)

Hydrograph of  developed area with High-Impact development  (Mannings N  of  0.04)

Flood extent, still need to define channel geometries or much smaller cells